## Project Title: Secure Multi-VLAN Enterprise Branch Network

This project simulates a corporate office with distinct departments, restricted access, and a secure connection to the internet.

your_secure_password

your_ssh_password

### 1. Key Features to Implement

To make this "LinkedIn-ready," you should incorporate these specific technologies:

- VLAN Segmentation: Divide the network into Management, Engineering, HR, and Guest segments.

- Inter-VLAN Routing: Use a "Router-on-a-Stick" configuration or a Layer 3 Switch.

- Redundancy (STP): Configure Rapid PVST+ to ensure the network stays up if a link fails.

- DHCP Server: Set up a centralized server (or router-based DHCP) to assign IP addresses automatically.

- Security (Access Control Lists): Create Standard and Extended ACLs to prevent the Guest VLAN from reaching the Engineering servers.

- Port Security: Lock down switch ports to specific MAC addresses to prevent unauthorized devices from plugging in.

- SSH Management: Disable Telnet and enforce SSH for all device configurations.

### 2. The Setup Checklist

Since you need a record of everything for your documentation, here is your initial "Shopping List" of components in Packet Tracer:

| Category | Device Recommendation | Purpose |
| --- | --- | --- |
| Edge Router | ISR 4321 | Handles NAT and the Firewall (ACLs). |
| Core Switch | 3650 (Layer 3) | Handles Inter-VLAN routing and high-speed switching. |
| Access Switch | 2960 Series | Connects the end-user PCs and implements Port Security. |
| End Devices | PCs, Laptops, Server | To test connectivity and demonstrate DHCP/ACLs. |

### 3. Step-by-Step Execution Plan

- Phase 1: Physical Topology. Place your devices and connect them using straight-through and crossover cables. Label everything clearly.

- Phase 2: Basic Config. Assign hostnames, set secret passwords, and enable SSH.

- Phase 3: Layer 2. Configure VLANs, assign ports, and set up 802.1Q Trunking between switches.

- Phase 4: Layer 3. Configure the Gateway (SVI or Sub-interfaces) and set up NAT so internal PCs can "ping" an external 8.8.8.8 address.

- Phase 5: Hardening. This is where your Security+ shines. Apply ACLs, Port Security, and disable unused ports.

### 4. How to Showcase This on GitHub

For a Junior Admin role, your GitHub shouldn't just have the .pkt file. It needs a README.md that includes:

- A Network Diagram: Export a screenshot of your Packet Tracer workspace.

- IP Addressing Scheme: A table showing your Subnets, Gateway addresses, and VLAN IDs.

- Verification Steps: Screenshots of successful "Pings" and show access-lists outputs to prove your security rules work.

### Project Recap & Records

As per your request, I will start the record for this project now. We will update this as we progress.

- Project Name: Secure Multi-VLAN Enterprise Branch

- Software Used: Cisco Packet Tracer 8.x

- Configurations Initiated: * VLAN Planning: Defined segments for Management (VLAN 10), Staff (VLAN 20), and Guests (VLAN 30).

  - Security Standards: Identified the need for SSH (Port 22) over Telnet (Port 23) and planned the implementation of Port Security on Access Switches.

- Explanation: These steps establish the "Least Privilege" principle at the network layer, ensuring that users only have access to the resources required for their specific roles.

Phase 2 (Basic Configuration & Security Hardening).

In this phase, we moved from having a "dumb switch" (out of the box) to a Secured Management Plane. This ensures that only authorized administrators (like you, Paige) can access the hardware to make changes.

Phase 2 Summary: Secure Management Setup

1. Identity and System Housekeeping

First, we gave the device a name and secured the configuration file itself.

- Hostname: Set the switch name to Paige-Core-SW. This is vital for logging and remote access so you know exactly which device you are logged into.

- Password Encryption: We enabled service password-encryption. By default, Cisco shows some passwords in plain text. This command scrambles them so a "shoulder surfer" or someone looking at a backup file can't read them.

2. The "Enable" Secret (Privileged Access)

We set an enable secret. In your Security+ studies, this represents Access Control.

- It protects the "Privileged EXEC" mode (the # prompt).1 Without this password, a user can look at basic stats but cannot change any configurations or view the sensitive running config.

3. Transitioning from Telnet to SSH

This is the most important "Junior Admin" task for security.

- The Problem: Telnet sends your password across the wire in plain text.2 Anyone with a packet sniffer (like Wireshark) could steal your credentials.

- The Solution: We generated RSA Encryption Keys. This allows the switch to use SSH (Secure Shell), which creates an encrypted tunnel for your management session.

- Requirement: To generate these keys, we had to define an ip domain-name (e.g., lab.local), as the switch uses the hostname and domain name to create the unique key.3

4. Virtual Line (VTY) Hardening

We configured the "Virtual Teletype" lines (the logic ports used for remote login).

- Transport Input SSH: We specifically told the switch to deny Telnet and only allow SSH.

- Login Local: We told the switch to check its own internal database for a username and password, rather than using a single, shared line password. This provides Accountability (knowing who logged in).

Record of Commands & Installations

| Task | Command(s) Used | Explanation |
| --- | --- | --- |
| Naming | hostname Paige-Core-SW | Unique identifier for the device. |
| Obfuscation | service password-encryption | Protects passwords from being read in the configuration file. |
| Privilege Security | enable secret [password] | Encrypts the gateway to "Global Configuration Mode." |
| Encryption Keys | crypto key generate rsa | Creates the mathematical keys needed to encrypt SSH traffic. |
| Remote Access | transport input ssh | Disables insecure Telnet (Port 23) in favor of SSH (Port 22). |

Phase 3 Checkpoint (The "Trunk" Recap)

Since we also just spent a lot of time on the connection between switches, here is the summary of that "Trunk" logic:

- We identified the physical ports (Fa0/1 on Core and Fa0/4 on Access).

- On the 3650 Core, we had to define the "Language" (encapsulation dot1q) before the switch would allow the port to become a trunk.

- We verified it using show interfaces trunk to ensure the "Status" was Trunking.

That is a major milestone! If both PCs now have 192.168.x.x addresses, you have successfully built a functional Layer 2 and Layer 3 infrastructure.

You have managed to:

- Secure the management plane (SSH/Passwords).

- Segment the network (VLANs).

- Build a High-Speed Trunk (Gigabit Uplink).

- Enable Inter-VLAN Routing (SVIs).

- Automate IP Addressing (DHCP).

1. Micro-Segmentation with Extended ACLs

In the previous phases, your VLANs were connected, but any user could talk to any other user. In Phase 5, we implemented Access Control Lists (ACLs).

- The Logic: We created an "Extended" ACL. Unlike Standard ACLs (which only look at the source IP), Extended ACLs can look at both the source and the destination.1

- The Rule: We specifically denied the Guest network (192.168.30.0) from reaching the Engineering network (192.168.10.0) while using a permit ip any any statement at the end to ensure they could still reach the internet.

- Security+ Concept: This demonstrates The Principle of Least Privilege—users should only have access to what they need for their job.

2. Layer 2 Port Security

We moved to the "Edge" of the network (the Access Switch) to prevent physical tampering.

- Sticky MAC Addresses: We configured the switch to "learn" the MAC address of the HR PC. Once learned, it is written into the running configuration.

- Maximum Limit: We restricted the port to only one MAC address.2

- Violation Mode (Shutdown): We set the policy so that if an unauthorized device (like a rogue laptop) is plugged in, the port immediately enters an "error-disabled" state (turns off).

- Security+ Concept: This is a form of NAC (Network Access Control) and prevents "MAC Spoofing" or unauthorized physical access.3

3. Attack Surface Reduction

We performed "Network Hygiene" by disabling what we weren't using.

- Interface Range Shutdown: We identified all ports on the 2960 switch that were not currently connected to a device and used the shutdown command.

- Security+ Concept: Reducing the Attack Surface. An unused port that is "Live" is a vulnerability. By shutting them down, you ensure an intruder cannot simply walk into an empty office and plug into the network.

Record of Commands & Installations for Portfolio

| Security Feature | Command Highlight | Purpose |
| --- | --- | --- |
| Traffic Filtering | ip access-list extended [NAME] | Controls the flow of data between internal departments. |
| ACL Application | ip access-group [NAME] in | Activates the filter on the specific VLAN Gateway (SVI). |
| Port Locking | switchport port-security mac-address sticky | Bonds a physical switch port to a specific device's hardware ID. |
| Threat Mitigation | switchport port-security violation shutdown | Automatically disables a port during a security breach. |
| System Hardening | interface range ... / shutdown | Closes unused entry points into the network infrastructure. |

Final Project Recap (Everything Installed & Configured)

To fulfill your requirement for a record of the entire project:

- Installed: Cisco Packet Tracer 8.x Environment.

- Updated: Configured the 3650 Core Switch to use dot1q trunking (updating its default behavior).

- Configured: * Hostnames/Passwords: Paige-Core-SW and Access-SW-1.

  - Remote Management: SSH (RSA 2048-bit keys).

  - Layer 2: VLANs 10, 20, 30 and Gigabit Trunking.

  - Layer 3: SVIs for Inter-VLAN routing and DHCP Pools for automated IP assignment.

  - Security: Extended ACLs and Sticky Port Security.